Objective:

Detect high-confidence permafrost zones using:

- Satellite-derived thermal data (MODIS LST)

- Vegetation/snow indicators (NDVI, NDSI)

- Terrain (DEM, slope, aspect)

- Soil and climate parameters (bulk density, FDD, TDD)

- And then visualize the final permafrost probability map with place names overlaid

Step-by-Step Functionality:

1. Load Input Data

- Reads raster files: DEM, LST, NDVI, Slope, NDSI, Soil properties, FDD, and TDD.

- Resamples them to a common spatial resolution and shape using the DEM as reference.

2. Clean FDD and TDD

- Ensures all invalid values (NaN, inf, negative) are handled.

- Resizes if shapes don’t match.

3. Estimate Soil Thermal Properties

- Computes frozen (Kf) and thawed (Kt) thermal conductivity using clay and bulk density.

- Clips them to realistic physical ranges.

4. Estimate n-factors

- Surface energy balance correction using NDVI and NDSI:

  - nf for freezing

  - nt for thawing

5. Compute TTOP (Temperature at the Top of Permafrost)

Where MAAT is approximated from LST.

6. Compute ALT (Active Layer Thickness)

Where:

- k = average thermal conductivity

- Lf = latent heat of fusion of water

- ρ = soil density

7. Classify High Confidence Permafrost Zones

- Zones where ALT < 1.5 m are marked as permafrost.

8. Compute Final Permafrost Probability

Combines three models:

9. Export Outputs

- ALT.tif: Estimated active layer thickness

- Permafrost_Probability.tif: Final combined permafrost probability map

- PermafrostZone_ALT_lt_1_5.tif: Binary mask of high-confidence permafrost

References:

1. TTOP (Temperature at the Top of Permafrost) Equation

Reference:

- Smith and Riseborough (2002). “Climate and the limits of permafrost: a zonal analysis.” Permafrost and Periglacial Processes.

- This equation relates Mean Annual Air Temperature (MAAT), thermal conductivities of soil, and n-factors to estimate the temperature at the top of permafrost.

2. Active Layer Thickness (ALT) Equation

Reference:

- Lunardini (1981). “Heat Transfer in Cold Climates.”

- This is based on the Stefan equation, which estimates the depth of seasonal thaw in frozen ground using:

  - Thermal conductivity k

  - Freeze Degree Days (FDD)

  - Soil density ρ

  - Latent heat of fusion L_f

3. Thermal Conductivity Estimation (Kf, Kt)

Reference:

- Modified from empirical formulations by Zhang et al. (2008) and Riseborough et al. (2008)

- These equations approximate frozen and thawed soil conductivity using bulk density and clay content, which are key factors influencing soil thermal behavior.

4. n-Factor Estimation

nf = np.where(NDVI > 0.2, 0.7, 0.5)

nt = np.where(NDSI > 0.3, 0.6, 0.9)

Reference:

- Klene et al. (2001); Zhang et al. (1997)

- Empirical assignment of freezing (nf) and thawing (nt) n-factors based on vegetation (NDVI) and snow (NDSI). These affect the surface energy balance.

5. MAGT Proxy Equation

Coefficients used:
α = -1.5, β = -2.2, δ = -0.5, ε = -0.2, C = 5.0

Reference Basis:

- Derived from methods in:

  - Obu et al. (2019), Northern Hemisphere permafrost map

  - Kumar et al. (2021), Permafrost modeling in the Indian Himalayas

- This equation combines normalized environmental variables to approximate MAGT where borehole data is unavailable.

6. Permafrost Probability

Reference:

- Composite logic adapted from Obu et al. (2019) and Raza Khan et al. (2021) for probability mapping.

- It integrates thermal (TTOP, MAGT) and physical (ALT) indicators for robust classifications

Output:

| Color | Interpretation |
| --- | --- |
| Bright Yellow/White (0.9+) | Very high permafrost probability (most likely frozen all year) |
| Orange to Purple (~0.6 – 0.8) | Moderate to high likelihood – likely continuous or discontinuous permafrost |
| Dark Purple (< 0.5) | Low or no permafrost – could be seasonally thawing or warm terrain |

Observation:

- Most terrain around Leh and Kargil is bright yellow, meaning high permafrost potential.

- Manali region shows relatively lower intensity (orange–purple), indicating transitional or patchy permafrost.

| Parameter | Explanation | Meaning |
| --- | --- | --- |
| FDD | Freeze Degree Days | Min = 0.0, Max = 5000.0, Mean = 4629.1 → shows the total freezing thermal load over the year. A high mean suggests long and cold winters, suitable for permafrost formation. |
| rho | Soil density (kg/m³) | Mean = 111725 → This seems 100x higher than typical (~1300–1600 kg/m³), likely because bulk_density was already in kg/m³ but was multiplied again. This high density artificially reduces ALT thickness. |
| k | Thermal conductivity (W/mK) | Range is realistic: 1.0–2.0 W/mK → indicates moderate to high conductivity depending on soil type. |
| ALT stats | Active Layer Thickness (m) | Max = 1.39 m, Mean = 0.0006 m → The extremely low mean means most regions are predicted to be near-frozen year-round, i.e., likely permafrost. |
| Permafrost pixel count | Number of pixels where ALT < 1.5 m | 277,215,314 → Almost all pixels in your study area are considered permafrost under this condition. |